# P-999 Quarterly Preventive Maintenance Procedure

## Section A: Seal Inspection

1. Isolate the P-999 pump using the standard Lockout/Tagout (LOTO) procedure.

2. Drain the fluid from the primary casing using the lower bleeder valve.

3. Remove the mechanical seal gland plate using a 14mm socket wrench.

4. Inspect the primary O-rings. If there is any sign of flattening or chemical degradation, replace them with Viton-grade O-rings (Part # VT-443-B).

## Section B: Bearing Lubrication and Replacement

The P-999 uses oil-bath lubrication. The oil must be drained and replaced every 3 months using ISO VG 68 synthetic oil.

If vibration analysis indicates a fault, the thrust bearings must be replaced. The accepted replacement part is the SKF-7720 angular contact ball bearing. Do not substitute with standard radial bearings as they will fail under the P-999's axial load.